# Day 1 — logic vs wire vs reg

This is one of the most common SystemVerilog interview questions.

# Why do we need them?

Signals inside a design need a data type.

Example:

logic a;

logic b;

logic y;

Here a, b, and y store values 0, 1, X, or Z.

# 1. wire (Verilog)

A wire represents a connection between components.

It cannot store a value by itself.

wire y;

assign y = a & b;

### Example

module and_gate(

input a,

input b,

output wire y

);

assign y = a & b;

endmodule

### Key Point

✅ Used with continuous assignment (assign)

❌ Cannot be assigned inside always block

always @(*)

begin

y = a & b;   // Error if y is wire

end

# 2. reg (Verilog)

A reg can hold a value assigned inside an always block.

reg y;

always @(*)

begin

y = a & b;

end

### Important

Many beginners think:

reg = Flip-Flop

This is WRONG.

reg simply means the signal is assigned inside an always block.

Whether hardware becomes combinational or sequential depends on the code.

# 3. logic (SystemVerilog)

SystemVerilog introduced logic.

logic replaces most uses of both wire and reg.

logic y;

Can be used in:

### Continuous Assignment

logic y;

assign y = a & b;

### always_comb

logic y;

always_comb

begin

y = a & b;

end

### always_ff

logic q;

always_ff @(posedge clk)

begin

q <= d;

end

# Comparison Table

| Feature | wire | reg | logic |
| --- | --- | --- | --- |
| Verilog | ✅ | ✅ | ❌ |
| SystemVerilog | ✅ | ✅ | ✅ |
| Used with assign | ✅ | ❌ | ✅ |
| Used in always block | ❌ | ✅ | ✅ |
| Recommended today | ❌ | ❌ | ✅ |

# Modern DV Coding Style

Instead of:

reg q;

reg y;

Use:

logic q;

logic y;

Most DV engineers use logic almost everywhere.

# Interview Questions

### Q1: Why was logic introduced?

Answer: To remove confusion between wire and reg and provide a single data type that works in most situations.

### Q2: Can a wire be assigned inside an always block?

Answer: No.

### Q3: Can logic be assigned inside always_comb?

Answer: Yes.

### Q4: Does reg always create a flip-flop?

Answer: No. It depends on the coding style.

# Day 1 Practice

### Task 1

Create a 2-input AND gate using:

assign

### Task 2

Create the same AND gate using:

always_comb

### Task 3

Create a D Flip-Flop using:

always_ff

# Output Expected

### AND Gate

| a | b | y |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

### D Flip-Flop

At every positive edge of clock:

d = 0 -> q = 0

d = 1 -> q = 1

### Day 1 Challenge

Write the code for:

- AND Gate using assign

- AND Gate using always_comb

- D Flip-Flop using always_ff

Send your code, and I'll review it like a DV interviewer.