Day 2 — always_comb, always_ff, and always_latch

These are SystemVerilog improvements over the old Verilog always @(*) style.

Why Were They Introduced?

In Verilog:

Always @(*)

Begin

Y = a & b;

End

The compiler doesn’t know whether this block is:

Combinational Logic?

Flip-Flop?

Latch?

SystemVerilog introduced:

Always_comb

Always_ff

Always_latch

To make your intention clear.

- Always_comb

Used for combinational logic.

Example

Module and_gate(

Input logic a,

Input logic b,

Output logic y

);

Always_comb

Begin

Y = a & b;

End

Endmodule

Hardware

A ----\

AND ---- y

B ----/

No memory element.

Output changes immediately when input changes.

Good Example

Always_comb

Begin

If(sel)

Y = a;

Else

Y = b;

End

Bad Example

Always_comb

Begin

If(sel)

Y = a;

End

Why bad?

When sel=0, y keeps its old value.

This creates a latch.

- Always_ff

Used for sequential logic (Flip-Flops).

Example

Module dff(

Input logic clk,

Input logic d,

Output logic q

);

Always_ff @(posedge clk)

Begin

Q <= d;

End

Endmodule

Hardware

D -| D  Q |-- q

| FF   |

Clk ->|>     |

Output updates only on clock edge.

Why use <= ?

Inside always_ff use:

<=

(non-blocking assignment)

Correct:

Q <= d;

Avoid:

Q = d;

- Always_latch

Used when you intentionally want a latch.

Example

Always_latch

Begin

If(enable)

Q <= d;

End

Hardware

Enable = 1 -> q follows d

Enable = 0 -> q stores previous value

Comparison Table

Feature	always_comb	always_ff	always_latch

Combinational Logic	✅	❌	❌

Flip-Flop	❌	✅	❌

Latch	❌	❌	✅

Clock Required	❌	✅	❌

Interview Questions

Q1

What is the replacement for:

Always @(*)

Answer:

Always_comb

Q2

Which block is used for D Flip-Flops?

Answer:

Always_ff

Q3

Why is always_comb better than always @(*)?

Answer:

Automatically checks combinational behavior.

Better error checking.

Cleaner and safer coding style.

Q4

Which assignment operator is preferred inside always_ff?

Answer:

<=

(non-blocking assignment)

Day 2 Practice

Task 1

Create a 2:1 MUX using:

Always_comb

Task 2

Create a D Flip-Flop using:

Always_ff

With:

Clk

Rst

D

Q

Task 3

Create a latch using:

Always_latch

Mini Interview Challenge

Predict the output:

Always_comb

Begin

If(sel)

Y = a;

End

When:

Sel = 1, a = 1  => y = ?

Sel = 0         => y = ?

Answer:

When sel=0, y keeps its previous value, which creates a latch. This is usually a bug in combinational logic.

Tomorrow (Day 3) we’ll learn SystemVerilog Data Types:

Bit

Logic

Byte

Int

Integer

These are frequently asked in DV fresher interviews.